14 April 2005

Review of CONCAWE Data for Crude Oil Pipelines
Calculation of Failure Rates to end of 2003

1	Summary

This review has been carried out for Crude Oil pipelines using CONCAWE data.  The methodology for assessing pipeline failures and hole sizes from reported CONCAWE data is similar to that used in an analysis carried out by W S Atkins in 1997 [Ref. 1].

2	Results and Conclusions

The failure rate of Crude Oil Pipelines for CONCAWE for the period 1971 to 2003 inclusive has been assessed as follows:-

Failure Rates per 1000 kilometre-years

| Spillage Cause | Pinhole | Hole | Rupture | Total |
| --- | --- | --- | --- | --- |
| Mechanical | 0.015 | 0.040 | 0.026 | 0.080 |
| Corrosion | 0.036 | 0.047 | 0.007 | 0.091 |
| Natural | 0.000 | 0.007 | 0.007 | 0.015 |
| Third Party | 0.040 | 0.077 | 0.029 | 0.146 |
| Total | 0.091 | 0.171 | 0.069 | 0.332 |

91 relevant spillage events have been analysed for the 33-year period.  These failure rates represent the average failure rates for European crude oil pipelines. The results appear to be a realistic assessment of the available data, and give failure rates that are similar to other international databases for pipeline failure such as the EGIG database.

3	Introduction

CONCAWE Oil Pipelines Management Group’s Special Task Force on Oil Pipeline Spillages publishes an annual report reviewing the performance of cross-country pipelines in Western Europe.  Based on the statistics given in these reports it has been possible to synthesise the characteristic pipeline failure rate for crude oil pipelines in Europe.

Most of the data used in this analysis has been derived using the full CONCAWE listing of failures published in the CONCAWE publication “Western Europe cross-country oil pipelines 30-year performance statistics” [Ref. 2]. In addition, spillage data for 2001, 2002 and 2003 have been obtained from later annual reports. However, some key information is not available in this report, and this further information has been obtained directly from CONCAWE.

The analysis methodology in this report, which is applied to derive the pipeline failure cases and the hole sizes is similar to that used in an analysis carried out by W S Atkins in 1997 [Ref. 1]. All results have been tabulated and fully described so that they can be replicated if required

4	Assumptions in this Analysis

Crude oil pipeline failures are categorised into four main types

Mechanical – covering construction and material defects

Corrosion – covering internal and external corrosion

Natural  - covering ground movement, land movement and floods

Third Party – covering accidental, deliberate and incidental 3rd party damage

For the few cases of operational failure, an alternative (usually mechanical or third party) is assigned as the secondary cause.

Several failure events are not included because they do not apply to line pipe. Failures excluded from the CONCAWE data-sets are:-  Valve equipment, Flanges, bolts etc,  Gaskets / O-ring materials, Instrument fittings, Couplings / branches, and Pumping stations etc.   The total number of crude oil pipeline failures recorded between 1971 and 2003 is 124, but 33 of these involve other equipment, so 91 failures are included in this analysis.  Appendix 1 shows the annual number of pipeline failures and those excluded.

The criteria for assigning hole sizes was as follows:-

| Failure Mechanism | Rupture | Puncture | Pinhole |
| --- | --- | --- | --- |
| Third party activity | > 250 m3 | 11 to 250 m3 | 10 m3 or less |
| Corrosion | > 500 m3 | 11 to 500 m3 | l10 m3 or less |
| Mechanical | > 200 m3 | 11 to 200 m3 | 10 m3 or less |
| Natural | > 200 m3 | 11 to 200 m3 | 10 m3 or less |

CONCAWE [Ref. 2] reports that of the 379 pipeline spillages recorded for European pipelines for both crude and clean product oils in the period 1971 to 2002, 176 have the hole size recorded.  The CONCAWE analysis shows that the relationship between and amount spilled and recorded hole size is broadly as expected, so the analysis method described above is used.

5	Pipeline Exposure

Data obtained from CONCAWE:- Average annual kilometres of clean product pipeline in Europe for the period 1971 to 2003 is 8,316 kilometres.

Therefore, the total exposure for the 33-year period 1971-2003 inclusive is

33 x 8,316 = 274,428 kilometre-years

6	Results

Detailed tables showing the year-by-year failures are attached in Appendix 2.

The main Results are as follows:-

Table 1 - Spillages from Various Pipeline Diameters

| Pipeline Diameter | Number of spillages | Percentage |
| --- | --- | --- |
| >30 inch | 10 | 11% |
| 24-30 inches | 9 | 9.9% |
| 16-24 inches | 26 | 28.6% |
| 12-16 inches | 11 | 12.1% |
| 8-12 inches | 24 | 26.4% |
| less than 8 inches | 11 | 12.1% |

Table 2 – Spillages by Cause

| Spillage Cause | Number of spillages | Percentage |
| --- | --- | --- |
| Mechanical | 22 | 24.2% |
| Corrosion | 25 | 27.5% |
| Natural | 4 | 4.4% |
| Third Party | 40 | 44.0% |
| Total | 91 | 100% |

Table 3 – Spillages by Cause and Hole Size

| Spillage Cause | Pinhole | Hole | Rupture | Total |
| --- | --- | --- | --- | --- |
| Mechanical | 4 | 11 | 7 | 22 |
| Corrosion | 10 | 13 | 2 | 25 |
| Natural | 0 | 2 | 2 | 4 |
| Third Party | 11 | 21 | 8 | 40 |
| Total | 25 | 47 | 19 | 91 |

Table 4 – Failure Rate per 1000 kilometre-years – by Hole Size and Cause

| Spillage Cause | Pinhole | Hole | Rupture | Total |
| --- | --- | --- | --- | --- |
| Mechanical | 0.015 | 0.040 | 0.026 | 0.080 |
| Corrosion | 0.036 | 0.047 | 0.007 | 0.091 |
| Natural | 0.000 | 0.007 | 0.007 | 0.015 |
| Third Party | 0.040 | 0.077 | 0.029 | 0.146 |
| Total | 0.091 | 0.171 | 0.069 | 0.332 |

Table 5 – Failure Rates as Percentage of Total

| Spillage Cause | Pinhole | Hole | Rupture | Total |
| --- | --- | --- | --- | --- |
| Mechanical | 4.4% | 12.1% | 7.7% | 24.2% |
| Corrosion | 11.0% | 14.3% | 2.2% | 27.5% |
| Natural | 0% | 2.2% | 2.2% | 4.4% |
| Third Party | 12.1% | 23.1% | 8.8% | 44.0% |
| Total | 27.5% | 51.6% | 20.9% | 100% |

The overall failure rate is therefore 0.332 per 1000 kilometre-years.

R A McConnell
14 April 2005

References

1	“Pipeline Failure Model”, A Report prepared by W S Atkins Safety & Reliability for and on behalf of The Health & Safety Executive, Report Number AM5099 /430 / R8000 / WP1.00, Issue 01, dated August 1997

2	“Western European Cross-Country Oil Pipelines 30-year Performance Statistics”, prepared on behalf of CONCAWE oil pipelines Management Group (OPMG) by D. Lyons (Consultant), CONCAWE Report No 1/02

Appendix 2 – Analysis of Data